Descriptions of QT12-15 Block Machine:

“QT” in QT 12-15 is given by Chinese government that all of the Concrete Block machine must use this Series Number.

“12” in QT12-15 stands for the production quantity is 12 blocks "390×190×190mm" per mould,

“15” in QT12-15 stands for one cycle time is 15 seconds.

Since the moulds are changeable, the concrete equipment is able to produce different types of blocks/bricks, and this is a hydraulic machine can produces a stronger block, due to the greater pound pressure applied by the hydraulic system.

The block manufacturer can also realize substantial savings by using less cement. A ratio from 8:1 to 11:1 sand/cement can be achieved. This machine is designed for the powerful manufacturer with a steady year turnover and with a certain number casher to make the factory running, normally it is for a big project.

This machines can be equipped with an automatic elevator/skip for loading mix into hopper, agitator, an automatic pallet feeder, pallet ejector, brush for cleaning blocks and a run off conveyor (see above). It is suggested that you use a high capacity mixer of over 500 liters. A work force of 20 to 50 employees would keep this factory running at full capacity.

QT 12-15 Model Specification

| Hollow Block Capacity(390×190×190mm) | 12/module |
| --- | --- |
| Solid Brick Capacity(240×115×53mm) | 60/module |
| Holland Brick Capacity(240×115×90mm) | 30/module |
| Main Dimension(L×W×H) | 8200×2300×3000(mm) |
| Pallet Size(L×W×H) | 1310×840×40(mm) |
| Pressure Rating | 12~25Mpa |
| Vibration | 50~90KN |
| Vibration Frequency | 2900~5600r/min |
| Cycle Time | 15-20(s) |
| Power(Total) | 46kw |
| Weight | 13000kg |
| Hollow Block Capacity(390×190×190mm) | 19200-23040/8hours |
| Solid Brick Capacity(240×115×53mm) | 97920-115200/8hours |
| Holland Brick Capacity(200×100×60mm) | 45600-57600/8hours |

Optional Parts:

1. Skip type concrete mixer……………………. Model JS750

2. Mould:  we can manufacture the a certain special mould for you accordingly .

3. Double layer Color-face device…………………Model JW350

4. Batcher machine…………………………………Model PLD1600

Advantages Of QT Series Automatic Block Making Machine:

1. Perfect humanization Electrical control system

Siemens PLC with Visualization touch screen provide you a Child operation machine.

2. Frequency Conversional Control Technology:

It guarantees the precise and parallel movement between tamper head and mould, protects the products from transfiguration; increases the production rate by improving the vibrating efficiency, and Power saving: 30%;

3. Hydraulic System. Make in China, world Quality , and we can provide you the hydraulic motor as you request .

4. Compulsory Feeding system make sure that the density and intensity of the products are guaranteed;

5. Various product scope:

It produces various color-face bricks (layered material feeding),through-body tiles，lock linkage blocks, road curb bricks, hydraulic blocks、hollow blocks, perforated bricks, and standard bricks etc.