Descriptions of JD1800 Foam Concrete Machine:

“JD” in JD1800 is given by Chinese government that all of the Concrete Block machine must use this Series Number.

“1800” in JD1800 stands for the capacity of the mixer is "1.8m3"

Light concrete mixing machine is specially designed for Cellular foam concrete, adopted the no linner concrete mixer to make sure the not mixing is fast and homogenous.

Since the moulds are changeable, the concrete equipment is able to produce different types of blocks/bricks, and this is a machine can produces a stronger block, due to the greater foam agent. .

The block manufacturer can also realize substantial savings by using less cement. A ratio from 3:1 to 2:1 sand/cement can be achieved. This machine is designed for the manufacturer with limited space, limited budget and the desire to achieve good quality, high volume of blocks, with the minimum cash outlay.

This machines can be equipped with an automatic elevator/skip for loading mix into hopper, agitator, an automatic pallet feeder, pallet ejector, brush for cleaning blocks and a run off conveyor. It is suggested that you use a high capacity mixer of over 500 liters. A work force of 5 to 7 employees would keep this factory running at full capacity.

JD1800 Foam concrete Model Specification

| Model | JD1800 |
| --- | --- |
| Input volume | 1800L |
| Discharge volume | 1000L |
| Productivity | 8-12m3/h |
| Main shaft revolution | 32r/min |
| Mixing motor | 11kw |
| Skip motor | 5.5kw |
| Vibrate motor | 0.4kw |
| Foaming Machine | 5.5kw |
| Water pump motor | 1.1kw |
| Overall dimension | 2600*2250*2800mm |
| Total weight | 3200kg |
| Concrete Pump | HBT30 |
| Max. Grouting flow | 30 m³/hour |
| Max. Pressure | 6Mpa |
| Horizontal transporting range | 300 m |
| Vertical transporting height | 60 m |
| Max Aggregates Size | 8 mm |
| Pump head | 2 Pcs |
| Oil cylinder quantity | 2 Pcs |
| Oil cylinder diameter*stroke | Φ80*1000 mm |
| Material cylinder diameter*stroke | Φ140*1000 mm |
| Max. pressure of hydraulic | 18Mpa |
| Hydraulic Oil Tank | 200L |
| Motor power | 30 KW |
| Dimension (L*W*H) | 3100*920*1300 mm |
| Weight | 1900 kg |

Optional Parts:

1. Conveying pump ……………………. Model HBT30

2. Mould:  we can manufacture the a certain special mould for you accordingly.

3. Foaming agent.

Advantages Of JD Series Foam Concrete Making Machine:

1. Perfect humanization Electrical control system

Siemens PLC with Visualization touch screen provide you a Child operation machine.

2. Environment protection.

3. Hydraulic System. Make in China, world Quality, and we can provide you the hydraulic motor as you request .

4. Compulsory Feeding system make sure that the density and intensity of the products are guaranteed;

5. Various product scope:

It produces various bricks or wall panel, hollow blocks, perforated bricks, and standard bricks etc.