Descriptions of PLD Series Aggregate batcher Machine:

“PLD” in PLD800-2 is given by Chinese government that all of the Concrete Batching machine must use this Series Number.

“800” in PLD800-2 stands for the capacity of the weighing hopper is "800 liter"

“2” in PLD800-2stands for number of the storage hopper is 2.

The block or concrete manufacturer can also realize substantial savings by using less sand and stone to produce the high quality product.

Number and volume of hopper can design and produce according the concrete request.

This series batching machine adopted the accumulation weighing , the weighing accuracy is ±2%.

This series batching machine can be equipped with an automatic control or wired remote for loading aggregate into skip hopper.

Advantages Of PLD Series Aggregate batcher Machine:

1. Perfect humanization Electrical control system

Siemens PLC or remote control operation system provide you a accredited operation machine.

2. Load Cell :

It guarantees the precise of the aggregate and protects the products from transfiguration.

3. Discharging System. Make in China, world Quality, and we can provide you the rubber belt as you request .

4. Compulsory Feeding system make sure that the density and intensity of the products are guaranteed;

PLD800-3 Model Specification

| Item | Unit | PLD 800-2 | PLD 1200-3 | PLD 1600-4 |
| --- | --- | --- | --- | --- |
| Scale hopper volume | Liter | 800 | 1200 | 1600 |
| Storage hopper volume | m3 | 2×2 | 3×2.2 | 3×2.2 |
| Productivity | m3/h | ≥48 | ≥60 | ≥80 |
| Batching precision |  | ±2% | ±2% | ±2% |
| Max weighting | kg | 1500 | 2000 | 3000 |
| Aggregate number |  | 2 | 3 | 4 |
| Feeding height | mm | 3000 | 3000 | 3000 |
| General power | kW | 3×3 | 4×3 | 5×3 |
| Weight | kg | 2500 | 3900 | 5750 |
| Dimension (L*W*H) | mm | 6400×1600×3000 | 9600×2000×3200 | 12900×2000×3200 |
| Division value checking | kg | 2.0 | 2.0 | 2.0 |
| Optional Structure character | The feeding structure is arranged in line, charging through the belt, and the measuring system is a single sensor and lever. |  |  |  |