UTM-25 Mobile Concrete Mixing Plant

Descriptions of Mobile Concrete Mixing Plant

Mobile concrete mixing plant which adopts advanced technologies and scientific researching results from home and abroad and combines our company’s year’s mixing station producing experience, which is a new type mobile concrete mixing station designed by our company independently . It is now one of the best mixers in China, which boasts compact and reasonable designing. Most functions of the mixing station are finished in the pulling chassis, combining aggregate, electronic weighing, mixing and lifting together. It can be stopped to run or be pulled away easily, which is convenient to move and easy to install. It is can applied to various construction sites like road surface, bridges, dams and airport, and also like architectural workings, concrete product plant and commercial concrete production .

General Information of UTM-25 Mobile Concrete Mixing Plant

Capacity: 25 m3/h

Mixing Time: 60 s

Max Aggregate Size: 60/80 mm

Discharging Height: 3800 mm

Total Power: 35 kw

Total Weight: 20 Ton

Total Dimensions: 14500×4050×7600 mm

Technical Parameters of Mobile Concrete Mixing Plant UTM-25

1. MIXING SYSTEM

Twin Shaft Concrete Mixer: JS500

Volume:  800L

Capacity: 500L

Mixing Speed: 35 r/min

Mixing Blade: 2×7 pcs

Mixer Motor: 18.5kw

Weight: 3700kg

Dimensions: 3030x2300x2680 mm
2. AGGREGATE SYSTEM

2.1Aggregate Batcher          PLD800

Volume: 4×4m3=16 m3

Sort of aggregate:  4

Power: 7.5 kw

Sand vibrator: 0.37kw×2

2.2. Aggregate Belt Conveyor          1 unit

Width: 800 mm

Length: 18000 mm

Speed: 1.6 m/s

Motor: 7.5 kw

3. CEMENT SYSTEM (OPTIONAL)

Cement Silo: 20 Ton

Cement Screw Feeder: 1 unit

Diameter: Φ219mm

Length: 11.8 m
4. ADDITIVE SYSTEM (OPTIONAL)

Additive silo capacity: 1 m3

Pneumatic valve: Φ50 mm

Pump: 3 kw

5. WATER SYSTEM

Submersible pump: 3 kw

Water pump: 0.75 kw

Butterfly valve: Φ150 mm

Water tank: 3 m3

6. AIR SYSTEM

Air Compressor: 1 unit

Power: 7.5 kw

Pressure: 0.7 Mpa

Airflew: 0.8 m3/min
7. WEIGHING SYSTEM:

7.1 Aggregate weighing   1 unit

Weighing Capacity: 1000 kg

Precision:  ±1%

7.2 Cement weighing      1 unit

Weighing Capacity: 300 kg

Precision:  ±1%

7.3 Water Weighing       1 unit

Weighing Capacity: 200 kg

Precision: ±1%

7.4 Additive Weighing     1 unit

Weighing Capacity: 20 kg

Precision: ±1%

8. CONTROL SYSTEM:

PLC command and control operation .

Main electrical components : Schneider

17"colorful computer

Manuel and Auto Control is freely changeable .