UTM-75 Mobile Concrete Mixing Plant

Descriptions of UTM-75 Mobile Concrete Mixing Plant:

This series of mixing plant adopts the semi-pulling structure with running system and standard coupling pin included. It can be dragged by trailer. Without foundation and commissioning enables convenient transportation, the mixing plant can be divided into several standard modules, and then be packed into containers.

General Information of UTM-75 Mobile Concrete Mixing Plant:

Capacity: 75 m3/h

Mixing Time: 60 S

Max Aggregate Size: 60/80 mm

Discharging Height: 3800 mm

Total Power: 100 kw

Total Weight: 30 Ton

Total Dimensions: 200000×4200×8300 mm

Transportation: Double Axle Chassis

Technical Parameters of UTM-75 Mobile Concrete Mixing Plant

1. MIXING SYSTEM

Twin Shaft Concrete Mixer: JS1500

Volume: 2400 L

Capacity: 1500 L

Paddle quantity: 20pieces

Paddle Speed: 26 r/min

Mixing Motor: 2×30 kw

2. AGGREGATE SYSTEM

2.1Aggregate Batcher   PLD2400

Volume: 4×10m3=32

Sort of aggregate: 4

Capacity: 110 m3/h

Power: 11 kw

Sand vibrator: 0.37kw×2

2.2. Aggregate Belt Conveyor     1 unit

Width: 1000 mm

Length: 20000mm

Speed: 1.6 m/s

Motor: 11 kw

3. CEMENT SYSTEM

Cement Silo: 30 Ton

Cement Screw Feeder: 2 units

Diameter: Φ219mm

Length: 11.8 m

4. ADDITIVE SYSTEM

Additive silo capacity: 1 m3

Pneumatic valve: Φ50 mm

Pump: 3kw

5. WATER SYSTEM

Submersible pump: 3kw

Water pump: 0.75kw

Water tank: 3 m3

6. AIR SYSTEM

Air Compressor: 1 unit

Power: 7.5kw

Pressure: 0.7 Mpa

Airflew: 0.8 m3/min

7. WEIGHING SYSTEM:

7.1 Aggregate weighing         1 unit

Weighing Capacity: 1.6 m3

Precision: ±2%

7.2 Cement weighing        1 unit

Weighing Capacity: 1m3

Precision: ±1%

7.3 Water Weighing      1 unit

Weighing Capacity: 150 kg

Precision: ±1%

7.4 Additive Weighing    1 unit

Weighing Capacity: 40 kg

8. CONTROL SYSTEM:

PLC command and control operation

Main electrical components: Schneider

17’’ colorful computer

Manuel and Auto Control is freely changeable