UTM-35 Mobile Concrete Mixing Plant

Descriptions of UTM-35 Mobile Concrete Mixing Plant:

This series of mixing plant adopts the semi-pulling structure with running system and standard coupling pin included. It can be dragged by trailer. Without foundation and commissioning enables convenient transportation, the mixing plant can be divided into several standard modules, and then be packed into containers.

General Information of UTM-35 Mobile Concrete Mixing Plant:

Capacity: 35m3/h

Mixing Time: 60 S

Max Aggregate Size: 60/80 mm

Discharging Height: 3800 mm

Total Power: 60kw

Total Weight: 18Ton

Total Dimensions: 14500×4050×7600mm

Transportation: Double Axle Chassis

Technical Parameters of UTM-35 Mobile Concrete Mixing Plant:

1. MIXING SYSTEM

Twin Shaft Concrete Mixer: JS750

Volume: 1250 L

Capacity: 750 L

Paddle quantity: 16 pieces

Paddle Speed: 30.5r/min

Mixing Motor: 30kw

2. AGGREGATE SYSTEM

2.1Aggregate Batcher: PLD1200

Volume: 4×5m3=32

Sort of aggregate: 4

Capacity: 60 m3/h

Power: 7.5 kw

Sand vibrator: 0.37kw×2

2.2. Aggregate Belt Conveyor  1 unit

Width: 800 mm

Length: 18000mm

Speed: 1.6 m/s

Motor: 7.5 kw

3. CEMENT SYSTEM

Cement Silo: 20 Ton

Cement Screw Feeder: 2 units

Diameter: Φ219mm

Length: 11.8 m

4. ADDITIVE SYSTEM

Additive silo capacity: 1 m3

Pneumatic valve: Φ50 mm

Pump: 3 kw

5. WATER SYSTEM

Submersible pump: 3 kw

Water pump: 0.75 kw

Water tank: 3 m3

6. AIR SYSTEM

Air Compressor: 1 unit

Power: 7.5 kw

Pressure: 0.7 Mpa

Air flew: 0.8 m3/min

7. WEIGHING SYSTEM:

7.1 Aggregate weighing   1 unit

Weighing Capacity: 1.6 m3

Precision: ±2%

7.2 Cement weighing  1 unit

Weighing Capacity: 1m3

Precision: ±1%

7.3 Water Weighing     1 unit

Weighing Capacity: 150 kg

Precision: ±1%

7.4 Additive Weighing   1 unit

Weighing Capacity: 40 kg

Precision: ±1%

8. CONTROL SYSTEM:

PLC command and control operation

Main electrical components: Schneider

17’’ colorful computer

Manuel and Auto Control is freely changeable